# Worked Example: NI/SI Consumption-Based Emission Factor Calculation

Source databases, their structure, and how they are combined to produce one monthly figure.

# 0. Pipeline overview

Four raw datasets are built and maintained independently before any calculation. Each answers one question:

| Dataset | Question it answers | File(s) |
| --- | --- | --- |
| Generation_MD | How much electricity did each generator produce, half-hour by half-hour? | EA_EMI_raw/*_Generation_MD.csv (60 monthly files) |
| Dispatched generation plant (SCD2) | What is each thermal/geothermal plant's own emission factor? | EA data/Dispatched generation plant.csv |
| NSP Tables | Which island (NI or SI) is each generator's connection point in? | EA data/NSP Tables/*.csv (106 daily snapshots) |
| HVDC transfer | How much electricity crossed Cook Strait, and which direction? | EA_EMI_raw/EMI_HVDC_Official_2021-2025.csv |

The calculation joins these four in a fixed order: NSP gives island per generator → plant register gives EF per generator → Generation_MD gives MWh and CO₂ per generator, summed to island level → HVDC gives the inter-island trade → the trade is used to convert generation into consumption.

# 1. Database structures (built independently of the calculation)

## 1.1 Generation_MD — half-hourly metered generation

Source: Electricity Authority EMI, dataset Generation_MD. One file per calendar month, 2021–2025 (60 files). Each row is one generator (POC) × one fuel × one calendar day.

| Column | Meaning |
| --- | --- |
| POC_Code | Point of Connection code — identifies the generator's physical grid connection (e.g. HLY2201 = Huntly) |
| Fuel_Code | Fuel type (Hydro, Gas, Coal, Wind, Geo, ...) |
| Trading_Date | Calendar day |
| TP1 ... TP48 | Energy generated in each half-hour trading period that day, in kWh |

Used for: the volume of electricity actually generated. Nothing here says which island the POC is in, or what its emission factor is — both come from the other two databases below.

## 1.2 Dispatched generation plant register (SCD2) — plant emission factors

Source: Electricity Authority Data Products team (supplied directly, not from EMI). One row per plant unit per validity period — a new row is added whenever a unit's heat rate or emissions factor changes (an SCD2 design), so a unit can appear more than once with different effective_start/end windows.

| Column | Meaning |
| --- | --- |
| PointOfConnectionCode | Same POC code system as Generation_MD — the join key |
| FuelCode | Fuel type, used together with POC as the join key |
| HeatRate | Fuel energy used per unit of electricity output (kJ per kWh) |
| EmissionsFactor | CO₂ released per unit of fuel energy burned (g CO₂ per GJ) |
| effective_start_datetime / effective_end_datetime | Validity window — the row only applies to generation within this date range |

Used for: converting a generator's MWh output into kg CO₂, via:

EF_plant (kg CO2/MWh) = HeatRate (kJ/kWh) × EmissionsFactor (g CO2/GJ) ÷ 1,000,000

Only thermal/geothermal fuels (Coal, Gas, Diesel, Geothermal) have a non-null HeatRate/EmissionsFactor. Renewable fuels (Hydro, Wind, Solar) are not in this register for combustion purposes and are assigned EF = 0 directly in the calculation — they burn nothing.

## 1.3 NSP Tables — Network Supply Point List (POC → Island)

Source: Electricity Authority EMI report R_NSPL_DR. 106 daily snapshot files — the full list of supply points is re-published periodically, and each snapshot is kept because a POC's attributes (including which network region it sits in) can change over time.

| Column | Meaning |
| --- | --- |
| POC code | Join key — same code system as Generation_MD and the plant register |
| Island | NI or SI — the value needed for this analysis |
| Start date / End date | Validity window for this POC's listed attributes |

Used for: building a single POC → Island lookup table. All daily snapshots are concatenated and de-duplicated; if a POC appears with conflicting Island values across snapshots, the most recent Start date wins.

## 1.4 HVDC transfer — inter-island trade

Source: Electricity Authority EMI report “HVDC transfer”, half-hourly resolution, 1 Jan 2021 – 31 Dec 2025. This is the only one of the four datasets not about a generator — it measures the Cook Strait cable directly.

| Column | Meaning |
| --- | --- |
| Period start | Half-hour timestamp |
| Direction | “Northward at Haywards” (South Island → North Island) or “Southward at Benmore” (North Island → South Island) |
| Energy transfer (GWh) | Energy moved in that half-hour and direction (Southward values are recorded negative in the raw file; the script takes the absolute value) |

Used for: nothing about generation or emissions — purely the trade volume, kept entirely separate from the other three databases until the final balance step.

# 2. Calculation steps, with one real example carried through

Example generator: POC HLY2201, unit HLY5 (“Huntly 5”), fuel GAS, island NI. Example month: assume it generated 4,000 MWh in the month.

## Step 1 — Look up island (uses NSP Tables only)

HLY2201  →  Island = NI            (from NSP Tables, column ‘Island’)

## Step 2 — Look up plant emission factor (uses plant register only)

HeatRate = 7,300 kJ/kWh ;  EmissionsFactor = 52,800 g CO2/GJ
EF_plant = 7,300 × 52,800 ÷ 1,000,000 = 385.4 kg CO2/MWh

(matched on POC = HLY2201 AND FuelCode = GAS, for a row whose validity window covers the month in question)

## Step 3 — Compute this generator's emissions (uses Generation_MD + Step 2 result)

gen_mwh = sum(TP1..TP48 in kWh) / 1000, summed over the month = 4,000 MWh
co2_kg  = gen_mwh × EF_plant = 4,000 × 385.4 = 1,541,600 kg CO2

## Step 4 — Aggregate every generator to island level (Generation-side totals)

Repeat Steps 1–3 for every POC/fuel row that month, group by Island (from Step 1), and sum:

Generation_NI      = sum of gen_mwh for all POCs with Island = NI
CO2_generated_NI   = sum of co2_kg  for all POCs with Island = NI
EF_generation_NI   = CO2_generated_NI / Generation_NI

Illustrative monthly totals after this step: Generation_NI = 2,000,000 MWh, CO2_generated_NI = 300,000,000 kg → EF_generation_NI = 150 kg/MWh. Generation_SI = 1,300,000 MWh, all renewable → CO2_generated_SI = 0 → EF_generation_SI = 0 kg/MWh.

## Step 5 — Read the inter-island trade (uses HVDC file only, independently of Steps 1–4)

SI_to_NI = sum of ‘Northward at Haywards’ GWh that month × 1,000   = 400,000 MWh
NI_to_SI = sum of ‘Southward at Benmore’  GWh that month × 1,000   = 50,000 MWh
net_to_NI = SI_to_NI − NI_to_SI = 350,000 MWh

## Step 6 — Energy balance (combines Step 4 totals with Step 5 totals)

Consumption_NI = Generation_NI + net_to_NI = 2,000,000 + 350,000 = 2,350,000 MWh
Consumption_SI = Generation_SI − net_to_NI = 1,300,000 − 350,000 =   950,000 MWh

## Step 7 — Emissions balance (average-mix assumption)

Exported electricity is assumed to carry the exporting island's own average generation EF for the month (Step 4's EF_generation, not any individual plant's EF):

CO2_consumed_NI = CO2_generated_NI − NI_to_SI×EF_gen_NI + SI_to_NI×EF_gen_SI
               = 300,000,000 − (50,000×150) + (400,000×0)
               = 300,000,000 − 7,500,000 = 292,500,000 kg

CO2_consumed_SI = CO2_generated_SI − SI_to_NI×EF_gen_SI + NI_to_SI×EF_gen_NI
               = 0 − (400,000×0) + (50,000×150)
               = 7,500,000 kg

## Step 8 — Final consumption-based emission factor

EF_consumption_NI = CO2_consumed_NI / Consumption_NI = 292,500,000 / 2,350,000 = 124.5 kg/MWh
EF_consumption_SI = CO2_consumed_SI / Consumption_SI =   7,500,000 /   950,000 =   7.9 kg/MWh

Result: NI's consumption EF (124.5) is lower than its generation EF (150) because it imports clean SI hydro. SI's consumption EF (7.9) is above zero, despite 100% renewable generation, because it imports a little thermally-backed NI power.

# 3. Resolution and output

Steps 1–8 are repeated independently (not by averaging a finer series) at four resolutions — hourly, daily, monthly, annual — each built straight from the half-hourly source rows. Outputs: monthly_consumption_EF_NI_SI.csv, annual_consumption_EF_NI_SI.csv, daily_consumption_EF_NI_SI.csv, hourly_consumption_EF_NI_SI.csv, in Consumption/output/.

Built-in check: because trade only moves energy/emissions between islands, NI+SI generation must equal NI+SI consumption exactly, and likewise for CO₂. This is verified at every resolution (confirmed to 0.000000 difference).